# Gears Change Direction and Speed

Grade 6 Unit 2  •  Chapter 3

Learning goal  I can predict direction and relative speed in a simple gear train.

Two ordinary external gears touching each other rotate in opposite directions.

# Teeth connect motion

Their teeth engage to transmit rotation. If a small driver turns a larger driven gear, the larger gear turns more slowly. A large driver turning a smaller gear can make the smaller gear turn faster.

The tradeoff involves force effects as well as speed. Gears do not create extra energy; changing speed changes the torque available, with real systems also losing some energy.

Several external gear pairs and a three-gear arrangement on tabletop mounts. Gear teeth transmit rotation and establish speed relationships. Generated contextual illustration; not a scale model.

# Count and predict

Count teeth rather than judging size from a picture. For a simple pair, the same number of teeth pass the contact point on each gear.

An intermediate gear can change the output direction without changing the simple end-to-end ratio in an ordinary three-gear train. More complex arrangements need their full connections traced.

Use slow safe models. Mark a starting tooth and compare turns. State assumptions, such as external gears on separate axles, because internal gears or shared shafts follow different direction relationships.

# What to remember

Gear teeth transmit rotation and establish speed relationships.

Predictions depend on the actual mesh and axle connections.

# Check your understanding

Explain aloud, sketch, or write. Use evidence from the chapter.

1  Why does a large driven gear rotate more slowly than a small driver?

My thinking

2  Why should direction predictions identify whether gears share an axle or mesh externally?

My thinking

# Practise the words

Cover the meanings and explain the words. Then cover the words and name the meanings. Give an example.

gear  a toothed wheel transmitting motion

driver  the gear receiving input

driven gear  a gear moved by another gear

gear ratio  a comparison of tooth counts or rotation speeds

# Lesson connection

Lesson 3 · Gears

Original adaptation informed by Core Knowledge Foundation science materials including Simple Machines, HelpfulComputers, ProblemSolvingAndComputers. Shared under CC BY NC SA 4.0. Generated contextual illustration.